1. Which MAC address cannot be assigned to a host network adapter?

A. 00-02-03-04-05-06
B. 01-02-03-04-05-06
C. 02-03-04-05-06-07
D. 03-04-05-06-07-08

Answer: B

Multicast MAC addresses typically start like:

- 01:00:5E:xx:xx:xx (IPv4 multicast range)

- least significant bit (LSB) of the first octet is set to 1.

2. What is the default OSPF Router Dead Interval on a broadcast network?

A. 10 seconds
B. 20 seconds
C. 30 seconds
D. 40 seconds

Answer: D

3. Which OSPF packet is used to discover and maintain neighbor relationships?

A. DD
B. LSR
C. Hello
D. LSU

Answer: C

4. Which OSPF packet is used to request missing LSAs?

A. Hello
B. DD
C. LSR
D. LSAck

Answer: C

5. Which OSPF packet carries LSAs?

A. Hello
B. DD
C. LSU
D. LSAck

Answer: C

6. Which OSPF packet acknowledges LSU packets?

A. DD
B. LSAck
C. Hello
D. LSR

Answer: B

7. Which OSPF neighbor state indicates adjacency has been established?

A. Down
B. 2-Way
C. Exchange
D. Full

Answer: D

8. In which OSPF state is the Master/Slave relationship determined?

A. Init
B. 2-Way
C. ExStart
D. Full

Answer: C

9. Which of the following are stable OSPF neighbor states? (Multiple Choice)

A. Down
B. Attempt
C. 2-Way
D. Full

Answer: C, D

10. Which routing protocols are dynamic routing protocols? (Multiple Choice)

A. OSPF
B. BGP
C. Static Route
D. Direct Route

Answer: A, B

11. Which route type has the highest priority by default?

A. RIP   (AD = 100)
B. OSPF
C. Static Route
D. Direct Route

Answer: D

12. Which route type is preferred after Direct Route?

A. RIP
B. Static Route
C. OSPF
D. BGP

Answer: C

13. Which OSPF version supports IPv6?

A. OSPFv1
B. OSPFv2
C. OSPFv3
D. OSPFv4

Answer: C

14. Which network types require DR/BDR election? (Multiple Choice)

A. Broadcast
B. NBMA
C. Point-to-Point
D. Point-to-Multipoint

Answer: A, B

15. SNMP uses UDP as its transport protocol.

A. True
B. False

Answer: A

16. Which SNMP versions support encryption?

A. SNMPv1
B. SNMPv2c
C. SNMPv3
D. None

Answer: C

17. SNMP consists of which components? (Multiple Choice)

A. NMS
B. Agent
C. MIB
D. Managed Object

Answer: A, B, C, D

18. By default, SNMP traps are sent to which port?

A. 161
B. 162
C. 163
D. 164

Answer: B

Port 161 → Used for SNMP queries/GET/SET requests (manager to agent)

Port 162 → Used for SNMP traps (alerts sent from agent to manager)

19. Telnet uses which transport protocol?

A. TCP
B. UDP
C. ICMP
D. SCTP

Answer: A

20. What is the default Telnet port number?

A. 20
B. 21
C. 22
D. 23

Answer: D

21. What are valid Telnet authentication modes? (Multiple Choice)

A. AAA Authentication
B. Password Authentication
C. None Authentication
D. MD5 Authentication

Answer: A, B, C

22. The authentication mode must be configured before Telnet login is allowed.

A. True
B. False

Answer: A

23. Which VRP user interfaces are supported? (Multiple Choice)

A. Console
B. Telnet
C. SSH
D. Mini USB

Answer: A, B, C, D

24. Which command returns from System View to User View?

A. return
B. quit
C. exit
D. undo

Answer: A

25. What does the save command do in VRP?

A. Saves current configuration
B. Saves current time
C. Saves logs
D. Saves routing table

Answer: A

26. Which protocols are TCP-based? (Multiple Choice)

A. FTP
B. HTTP
C. TFTP   UDP based
D. Ping

Answer: A, B

27. Which protocol is NOT a file transfer protocol?

A. FTP
B. TFTP
C. SFTP
D. HTTP

Answer: D

28. Which port does FTP use for control connection?

A. 20
B. 21
C. 22
D. 23

Answer: B

29. PPPoE distinguishes sessions using:

A. MAC Address
B. Session ID
C. MAC Address + Session ID
D. IP Address

Answer: C

30. Which PPP protocol phase negotiates link parameters?

A. NCP
B. PAP
C. CHAP
D. LCP

Answer: D

31. During PPP LCP negotiation, which packet accepts proposed parameters?

A. Configure-Reject
B. Configure-Nak
C. Configure-Request
D. Configure-Ack

Answer: D

32. What is the VLAN ID field length in IEEE 802.1Q?

A. 8 bits
B. 10 bits
C. 12 bits
D. 16 bits

Answer: C

33. What is the valid VLAN ID range for users?

A. 0–4095
B. 1–4094
C. 1–4095
D. 2–4096

Answer: B

34. VLAN 0 can be assigned to users.

A. True
B. False

Answer: B

35. Which IEEE 802.11 standard supports only the 2.4 GHz band?

A. 802.11g
B. 802.11n
C. 802.11ac
D. 802.11ax

Answer: A

36. Which IEEE 802.11 standard supports both 2.4 GHz and 5 GHz?

A. 802.11a
B. 802.11g
C. 802.11n
D. 802.11ac

Answer: C

37. Which IEEE 802.11 standard works only in the 5 GHz band?

A. 802.11a
B. 802.11ac
C. Both A and B
D. 802.11g

Answer: C

38. WPA2-PSK is the only security policy supporting TKIP.

A. True
B. False

Answer: B

39. Which WLAN authentication methods are supported? (Multiple Choice)

A. Portal Authentication
B. MAC Authentication
C. 802.1X Authentication
D. RADIUS Authentication

Answer: A, B, C

40. What is the default bridge priority in STP?

A. 0
B. 4096
C. 32768
D. 65535

Answer: C

41. How many bits are used for Bridge Priority in STP?

A. 4
B. 8
C. 16
D. 32

Answer: C

42. How many bits are contained in STP Port ID?

A. 2
B. 4
C. 8
D. 16 ( 4 bits priority  + 12 bits port-num )

Answer: D

43. Which port roles were added by RSTP? (Multiple Choice)

A. Alternate Port
B. Backup Port
C. Root Port
D. Designated Port

Answer: A, B

44. Which states exist in RSTP? (Multiple Choice)

A. Discarding
B. Learning
C. Forwarding
D. Listening

Answer: A, B, C

45. Which state is NOT included in RSTP?

A. Learning
B. Discarding
C. Listening
D. Forwarding

Answer: C

46. Which protocol prevents Layer 2 loops?

A. NAT
B. STP/RSTP
C. OSPF
D. DHCP

Answer: B

47. Loops in Layer 2 networks can cause broadcast storms.

A. True
B. False

Answer: A

48. Which MPLS header fields exist? (Multiple Choice)

A. Label
B. EXP
C. TTL
D. S Bit

Answer: A, B, C, D

49. What does the S bit indicate in MPLS?

A. QoS Value
B. Bottom of Label Stack
C. Hop Count
D. Next Hop

Answer: B

50. Which planes exist in a network device? (Multiple Choice)

A. Data Plane
B. Control Plane
C. Management Plane
D. Service Plane

Answer: A, B, C

51. ICMP does not contain port numbers, therefore NAPT cannot be applied.

A. True
B. False

Answer: A

52. Which MAC prefix identifies multicast?

A. FF-FF-FF-FF-FF-FF
B. 01-00-5E-xx-xx-xx
C. 00-00-00-00-00-00
D. FE-80-xx-xx-xx-xx

Answer: B

53. What is the IPv6 basic header size?

A. 20 bytes
B. 32 bytes
C. 40 bytes
D. 64 bytes

Answer: C

54. IPv6 basic header length is fixed.

A. True
B. False

Answer: A

55. Which IPv6 header field is equivalent to IPv4 TTL?

A. Next Header
B. Hop Limit
C. Traffic Class
D. Flow Label

Answer: B

56. Which IPv6 field is used for QoS?

A. Traffic Class
B. Payload Length
C. Version
D. Next Header

Answer: A

57. Which Huawei Open APIs are supported? (Multiple Choice)

A. RESTful
B. NETCONF
C. OpenFlow
D. AAA

Answer: A, B, C

58. Which of the following is NOT a Huawei Open API?

A. RESTful
B. NETCONF
C. OpenFlow
D. AAA

Answer: D

59. Which command displays only static routes?

A. display ip routing-table
B. display ip routing-table protocol static
C. display route static
D. display route

Answer: B

60. Which are sources of routing table entries? (Multiple Choice)

A. Direct Routes
B. Static Routes
C. Dynamic Routes
D. Summary Routes

Answer: A, B, C, D